# Circuit Documentation

## Summary

This circuit is a multi-sensor fire and gas monitoring system that utilizes an ESP32 microcontroller to interface with various sensors, including gas sensors (MQ-4, MQ-5, MQ-7, MQ-135), a flame sensor (KY-026), a temperature and humidity sensor (DHT22), and a temperature sensor (TMP102). The system is designed to monitor environmental conditions and log data to a Firebase database. It also includes a buzzer and an LED for alerting purposes. The circuit is powered by a MOSFET-controlled power supply, and it uses an I2C multiplexer (TCA9548A) to manage multiple I2C devices.

## Component List

- MQ-5 Gas Sensor

  - Pins: VCC, GND, Digi Out, Analog out

  - Description: Detects natural gas and LPG.

  - Purpose: Provides analog and digital outputs for gas concentration.

- MQ-7 Gas Sensor

  - Pins: VCC, GND, AO, DO

  - Description: Detects carbon monoxide.

  - Purpose: Provides analog and digital outputs for CO concentration.

- MQ-4 Gas Sensor

  - Pins: A0, D0, GND, VCC

  - Description: Detects methane.

  - Purpose: Provides analog and digital outputs for methane concentration.

- MQ-135 Gas Sensor

  - Pins: Analog output, Digital Output, GND, Power

  - Description: Detects air quality.

  - Purpose: Provides analog and digital outputs for air quality.

- KY-026 Flame Sensor

  - Pins: A0, GND, VCC, D0

  - Description: Detects flame presence.

  - Purpose: Provides analog and digital outputs for flame detection.

- DHT22 Temperature and Humidity Sensor

  - Pins: GND, VCC, DAT

  - Description: Measures temperature and humidity.

  - Purpose: Provides digital output for temperature and humidity.

- TMP102 Temperature Sensor

  - Pins: GND, VCC, SDA, SCL, ALT, ADD0

  - Description: Measures temperature.

  - Purpose: Provides I2C interface for temperature data.

- TCA9548A I2C Multiplexer

  - Pins: 2SDA, 2SCL, 3SDA, 3SCL, 4SDA, 4SCL, 5SDA, 5SCL, 6SDA, 6SCL, 7SDA, 7SCL, VCC, GND, INPUTSDA, INPUTSCL, RESET, A0, A1, A2, 0SDA, 0SCL, 1SDA, 1SCL

  - Description: Manages multiple I2C devices.

  - Purpose: Allows multiple I2C devices to communicate with the ESP32.

- ESP32 Microcontroller

  - Pins: EN, VP, VN, D34, D35, D32, D33, GND, Vin, D23, D22, TX0, RX0, D21, D19, D18, D5, TX2, RX2, D4, D2, D15, 3V3

  - Description: Main processing unit.

  - Purpose: Interfaces with sensors and manages data logging.

- Buzzer

  - Pins: VCC, GND

  - Description: Provides audible alerts.

  - Purpose: Sounds an alarm when conditions are met.

- LED Two Pin (Red)

  - Pins: Cathode, Anode

  - Description: Visual indicator.

  - Purpose: Lights up to indicate an alert.

- MOSFET gate IRF9540N

  - Pins: Source (S), Drain (D), Gate (G)

  - Description: Controls power to the circuit.

  - Purpose: Acts as a switch for the power supply.

- MOSFET

  - Pins: Gate, Drain, Source

  - Description: General-purpose MOSFET.

  - Purpose: Used for switching applications.

- Resistor (220 Ohms)

  - Pins: pin1, pin2

  - Description: Limits current.

  - Purpose: Used in series with the LED.

- Resistor (10k Ohms)

  - Pins: pin1, pin2

  - Description: Limits current.

  - Purpose: Used in various parts of the circuit.

- Gear Potentiometer

  - Pins: SIG, VCC, GND

  - Description: Variable resistor.

  - Purpose: Provides adjustable resistance.

## Wiring Details

- MQ-5 Gas Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - Analog out connected to ESP32 pin D35.

- MQ-7 Gas Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - AO connected to ESP32 pin D32.

- MQ-4 Gas Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - A0 connected to ESP32 pin D34.

- MQ-135 Gas Sensor

  - Power connected to the common power rail.

  - GND connected to the common ground rail.

  - Analog output connected to ESP32 pin D33.

- KY-026 Flame Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - A0 connected to ESP32 pin D25.

  - D0 connected to ESP32 pin D4.

- DHT22 Temperature and Humidity Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - DAT connected to ESP32 pin D27.

- TMP102 Temperature Sensor

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - SDA connected to TCA9548A pin 0SDA.

  - SCL connected to TCA9548A pin 0SCL.

- TCA9548A I2C Multiplexer

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

  - INPUTSDA connected to ESP32 pin D21.

  - INPUTSCL connected to ESP32 pin D22.

- ESP32 Microcontroller

  - 3V3 connected to the common power rail.

  - GND connected to the common ground rail.

  - D23 connected to Buzzer VCC.

  - D26 connected to Resistor (220 Ohms) pin1.

  - D27 connected to DHT22 DAT.

  - D19 connected to MOSFET Gate.

  - D18 connected to MOSFET gate IRF9540N Gate (G).

- Buzzer

  - VCC connected to ESP32 pin D23.

  - GND connected to the common ground rail.

- LED Two Pin (Red)

  - Anode connected to Resistor (220 Ohms) pin2.

  - Cathode connected to the common ground rail.

- MOSFET gate IRF9540N

  - Source (S) connected to the common power rail.

  - Gate (G) connected to ESP32 pin D18.

- MOSFET

  - Gate connected to ESP32 pin D19.

  - Source connected to the common ground rail.

- Resistor (220 Ohms)

  - pin1 connected to ESP32 pin D26.

  - pin2 connected to LED Anode.

- Resistor (10k Ohms)

  - pin1 connected to ESP32 pin D27.

  - pin2 connected to the common power rail.

- Gear Potentiometer

  - SIG connected to ESP32 pin VP.

  - VCC connected to the common power rail.

  - GND connected to the common ground rail.

## Code Documentation

### ESP32 Code

The ESP32 code is responsible for interfacing with the sensors, processing the data, and communicating with a Firebase database. Below is a summary of the key functionalities:

- WiFi Setup: Connects to a specified WiFi network using the provided SSID and password.

- Firebase Communication: Utilizes HTTPClient to send data to Firebase, including historical and latest sensor readings.

- Sensor Reading: Reads data from various sensors, including gas sensors (MQ-4, MQ-5, MQ-7, MQ-135), flame sensor (KY-026), and temperature sensors (DHT22, TMP102).

- Auto-Calibration: Automatically calibrates the gas sensors in clean air conditions.

- Alarm System: Activates a buzzer and LED if sensor readings exceed predefined thresholds.

- Serial Commands: Processes serial commands for calibration, setting device ID, and clearing stored calibration data.

### ADS1115 Code

The ADS1115 code is a placeholder and does not contain any specific functionality. It includes empty setup and loop functions, indicating that the code is not yet implemented.

/* ------------------------------------------------------ ESP32 Multi-sensor Fire/Gas Monitor -> Firebase (Persistent logging) - MQ-4 (CH4), MQ-5 (NG/LPG), MQ-7 (CO), MQ-135 (Air quality) - KY-026 (flame) analog + digital - DHT22 - TMP102 via TCA9548A ch0 - ADS1115 via TCA9548A ch0 (optional) - Device ID persisted in Preferences (NVS). Can set via Serial "setid <ID>". - Auto-calibration of R0 on first run (clean air) or via "cal" Serial command. - Sends: POST -> /devices/<DEVICE_ID>/history.json (append) PUT -> /devices/<DEVICE_ID>/latest.json (latest snapshot) - Requires libraries: Adafruit_ADS1015 (or ADS1115), DHT - IMPORTANT: set WIFI_SSID/WIFI_PASS/FIREBASE_URL/FIREBASE_AUTH below ------------------------------------------------------ */ #include <WiFi.h> #include <WiFiClientSecure.h> #include <HTTPClient.h> #include <Wire.h> #include <Adafruit_ADS1015.h> #include "DHT.h" #include <Preferences.h> #include <time.h> Preferences prefs; Adafruit_ADS1115 ads; DHT dht(27, DHT22); // DHT on GPIO27 // ========== CONFIG - EDIT THESE ========== const char* WIFI_SSID = "YOUR_SSID"; const char* WIFI_PASS = "YOUR_PASS"; const char* FIREBASE_URL = "https://your-project-id.firebaseio.com"; // no trailing slash const char* FIREBASE_AUTH = "YOUR_DB_SECRET_OR_TOKEN"; // leave empty if not used const String PREF_NAMESPACE = "iot_cfg"; const String PREF_KEY_DEVICEID = "device_id"; const String PREF_KEY_R0_MQ4 = "r0_mq4"; const String PREF_KEY_R0_MQ5 = "r0_mq5"; const String PREF_KEY_R0_MQ7 = "r0_mq7"; const String PREF_KEY_R0_MQ135 = "r0_mq135"; String DEVICE_ID = "ESP32_UNKNOWN"; // default; will load from prefs if set // Publish interval const unsigned long PUBLISH_INTERVAL = 5000UL; // ms // Pins const int PIN_MQ5_ADC = 35; const int PIN_MQ7_ADC = 32; const int PIN_MQ4_ADC = 34; const int PIN_MQ135_ADC = 33; const int PIN_KY_A0 = 25; const int PIN_KY_D0 = 4; const int PIN_SIG_POT = 36; const int PIN_BUZZER = 23; const int PIN_LED = 26; const int PIN_HEATER_MOSFET = 18; const int PIN_MOSFET2 = 19; // ADC / voltage settings const float VCC_SENSOR = 5.0; const float ADC_REF = 3.3; const int ADC_RES = 4095; float VOLT_DIVIDER_RATIO = 0.666; // change if your divider different (e.g., 3.3/5 = 0.666) // RL values (ohm) - change if known from your module float RL_MQ135 = 10000.0; float RL_MQ7 = 10000.0; float RL_MQ4 = 20000.0; // guess; update if you know float RL_MQ5 = 20000.0; // guess; update if you know // Clean air ratios (Rs/R0) from datasheets (used for auto-calibration) const float CA_MQ4 = 4.4; const float CA_MQ5 = 6.5; const float CA_MQ7 = 27.0; const float CA_MQ135 = 3.6; // Curve params (log-log): log10(ppm) = m*log10(Rs/R0) + c struct Curve { float m; float c; }; Curve CURVE_MQ4 = { -0.318, 1.133 }; Curve CURVE_MQ5 = { -0.440, 1.800 }; Curve CURVE_MQ7 = { -0.770, 1.699 }; Curve CURVE_MQ135 = { -0.420, 1.920 }; // runtime R0 values float R0_MQ4 = NAN, R0_MQ5 = NAN, R0_MQ7 = NAN, R0_MQ135 = NAN; bool autoCalibrated = false; // helpers void tca_select(uint8_t ch) { if (ch>7) return; Wire.beginTransmission(0x70); Wire.write(1<<ch); Wire.endTransmission(); } float adcToVolt(int raw) { return ((float)raw / (float)ADC_RES) * ADC_REF; } float vModuleFromAdc(float vAdc) { if (VOLT_DIVIDER_RATIO <= 0.0001) return vAdc; return vAdc / VOLT_DIVIDER_RATIO; } float computeRsFromV(float vModule, float rLoad) { if (vModule <= 0.0001) return 1e9; return rLoad * ((VCC_SENSOR - vModule) / vModule); } float computePPM(float Rs, float R0, Curve curve) { if (!isfinite(R0) ¦¦ R0<=0 ¦¦ Rs<=0) return NAN; float ratio = Rs / R0; float logRatio = log10(ratio); float logppm = (logRatio - curve.c) / curve.m; float ppm = pow(10.0, logppm); return ppm; } // ========== Networking & Firebase helpers ========== String firebasePost(const String &path, const String &jsonPayload) { // POST -> path.json will create unique key (push) String url = FIREBASE_URL + path + ".json"; if (strlen(FIREBASE_AUTH) > 0) url += "?auth=" + String(FIREBASE_AUTH); WiFiClientSecure *client = new WiFiClientSecure(); client->setInsecure(); HTTPClient https; https.begin(*client, url); https.addHeader("Content-Type", "application/json"); int code = https.POST(jsonPayload); String resp = ""; if (code > 0) { resp = https.getString(); } else { Serial.printf("[HTTP] POST failed: %s\n", https.errorToString(code).c_str()); } https.end(); delete client; return resp; // contains response with name key if success } bool firebasePut(const String &path, const String &jsonPayload) { String url = FIREBASE_URL + path + ".json"; if (strlen(FIREBASE_AUTH) > 0) url += "?auth=" + String(FIREBASE_AUTH); WiFiClientSecure *client = new WiFiClientSecure(); client->setInsecure(); HTTPClient https; https.begin(*client, url); https.addHeader("Content-Type", "application/json"); int code = https.PUT(jsonPayload); https.end(); delete client; return (code >= 200 && code < 300); } // get ISO8601 timestamp using NTP (Malaysia UTC+08:00) void setupTime() { // set timezone for Malaysia: POSIX TZ is backwards: UTC+8 => "UTC-8" setenv("TZ", "UTC-8", 1); tzset(); configTime(0, 0, "pool.ntp.org", "time.google.com"); } String isoTimestamp() { time_t now = time(nullptr); if (now < 1600000000) return String(millis()); // no time yet struct tm timeinfo; localtime_r(&now, &timeinfo); char buf[32]; strftime(buf, sizeof(buf), "%Y-%m-%dT%H:%M:%S%z", &timeinfo); // e.g., 2025-11-19T18:00:05+0800 return String(buf); } // Serial command processing void processSerialCommands(String cmd) { cmd.trim(); if (cmd.length() == 0) return; if (cmd.equalsIgnoreCase("cal")) { Serial.println("Serial: Starting manual calibration (clean air required)..."); autoCalibrated = false; // force calibration } else if (cmd.startsWith("setid ")) { String id = cmd.substring(6); id.trim(); if (id.length()>0) { DEVICE_ID = id; prefs.begin(PREF_NAMESPACE.c_str(), false); prefs.putString(PREF_KEY_DEVICEID.c_str(), DEVICE_ID); prefs.end(); Serial.printf("Device ID set to: %s\n", DEVICE_ID.c_str()); } } else if (cmd.equalsIgnoreCase("showid")) { Serial.printf("Device ID: %s\n", DEVICE_ID.c_str()); } else if (cmd.equalsIgnoreCase("clearr0")) { prefs.begin(PREF_NAMESPACE.c_str(), false); prefs.remove(PREF_KEY_R0_MQ4.c_str()); prefs.remove(PREF_KEY_R0_MQ5.c_str()); prefs.remove(PREF_KEY_R0_MQ7.c_str()); prefs.remove(PREF_KEY_R0_MQ135.c_str()); prefs.end(); R0_MQ4 = R0_MQ5 = R0_MQ7 = R0_MQ135 = NAN; autoCalibrated = false; Serial.println("Cleared stored R0, next cycle will recalibrate."); } else { Serial.println("Unknown command. Commands: cal ¦ setid <ID> ¦ showid ¦ clearr0"); } } // Auto-calibration routine (clean air). Runs when R0 missing or forced. void tryAutoCalibrate() { if (autoCalibrated) return; // run only after a warm-up / wait static unsigned long started = millis(); if (millis() - started < 12000) return; // wait ~12s Serial.println("=== Starting MQ Auto-calibration (ensure CLEAN AIR) ==="); const int SAMPLES = 100; const int S_DELAY = 200; double sum4=0,sum5=0,sum7=0,sum135=0; int v4cnt=0,v5cnt=0,v7cnt=0,v135cnt=0; for (int i=0;i<SAMPLES;i++) { int r4 = readADCaveraged(PIN_MQ4_ADC,3,3); int r5 = readADCaveraged(PIN_MQ5_ADC,3,3); int r7 = readADCaveraged(PIN_MQ7_ADC,3,3); int r135 = readADCaveraged(PIN_MQ135_ADC,3,3); float v4 = vModuleFromAdc(adcToVolt(r4)); float v5 = vModuleFromAdc(adcToVolt(r5)); float v7 = vModuleFromAdc(adcToVolt(r7)); float v135 = vModuleFromAdc(adcToVolt(r135)); float Rs4 = computeRsFromV(v4, RL_MQ4); float Rs5 = computeRsFromV(v5, RL_MQ5); float Rs7 = computeRsFromV(v7, RL_MQ7); float Rs135 = computeRsFromV(v135, RL_MQ135); if (isfinite(Rs4) && Rs4<1e8) { sum4 += Rs4; v4cnt++; } if (isfinite(Rs5) && Rs5<1e8) { sum5 += Rs5; v5cnt++; } if (isfinite(Rs7) && Rs7<1e8) { sum7 += Rs7; v7cnt++; } if (isfinite(Rs135) && Rs135<1e8) { sum135 += Rs135; v135cnt++; } delay(S_DELAY); } prefs.begin(PREF_NAMESPACE.c_str(), false); if (v4cnt>0) { R0_MQ4 = (sum4 / v4cnt) / CA_MQ4; prefs.putFloat(PREF_KEY_R0_MQ4.c_str(), R0_MQ4); } if (v5cnt>0) { R0_MQ5 = (sum5 / v5cnt) / CA_MQ5; prefs.putFloat(PREF_KEY_R0_MQ5.c_str(), R0_MQ5); } if (v7cnt>0) { R0_MQ7 = (sum7 / v7cnt) / CA_MQ7; prefs.putFloat(PREF_KEY_R0_MQ7.c_str(), R0_MQ7); } if (v135cnt>0) { R0_MQ135 = (sum135 / v135cnt) / CA_MQ135; prefs.putFloat(PREF_KEY_R0_MQ135.c_str(), R0_MQ135); } prefs.end(); Serial.printf("Auto-cal done. R0s: MQ4=%.2f MQ5=%.2f MQ7=%.2f MQ135=%.2f\n", R0_MQ4, R0_MQ5, R0_MQ7, R0_MQ135); autoCalibrated = true; } // Setup void setup() { Serial.begin(115200); delay(200); Wire.begin(); // default SDA=21 SCL=22 dht.begin(); ads.begin(); // pins pinMode(PIN_BUZZER, OUTPUT); pinMode(PIN_LED, OUTPUT); pinMode(PIN_KY_D0, INPUT); pinMode(PIN_HEATER_MOSFET, OUTPUT); digitalWrite(PIN_HEATER_MOSFET, HIGH); // ADC attenuation analogSetPinAttenuation(PIN_MQ5_ADC, ADC_11db); analogSetPinAttenuation(PIN_MQ7_ADC, ADC_11db); analogSetPinAttenuation(PIN_MQ4_ADC, ADC_11db); analogSetPinAttenuation(PIN_MQ135_ADC, ADC_11db); analogSetPinAttenuation(PIN_KY_A0, ADC_11db); analogSetPinAttenuation(PIN_SIG_POT, ADC_11db); // preferences load prefs.begin(PREF_NAMESPACE.c_str(), false); DEVICE_ID = prefs.getString(PREF_KEY_DEVICEID.c_str(), DEVICE_ID).c_str(); R0_MQ4 = prefs.getFloat(PREF_KEY_R0_MQ4.c_str(), NAN); R0_MQ5 = prefs.getFloat(PREF_KEY_R0_MQ5.c_str(), NAN); R0_MQ7 = prefs.getFloat(PREF_KEY_R0_MQ7.c_str(), NAN); R0_MQ135 = prefs.getFloat(PREF_KEY_R0_MQ135.c_str(), NAN); prefs.end(); // WiFi Serial.printf("Connecting WiFi '%s' ...\n", WIFI_SSID); WiFi.begin(WIFI_SSID, WIFI_PASS); int tries = 0; while (WiFi.status() != WL_CONNECTED && tries < 40) { delay(250); Serial.print("."); tries++; } if (WiFi.status()==WL_CONNECTED) Serial.printf("\nWiFi connected: %s\n", WiFi.localIP().toString().c_str()); else Serial.println("\nWiFi failed - continuing offline"); // NTP/time setupTime(); Serial.printf("Device ID: %s\n", DEVICE_ID.c_str()); Serial.println("Serial commands: cal ¦ setid <ID> ¦ showid ¦ clearr0"); Serial.println("Startup complete."); } // Read averaged ADC int readADCaveraged(int gpioPin, int samples = 6, int delayMs = 3) { long sum=0; for (int i=0;i<samples;i++){ sum += analogRead(gpioPin); delay(delayMs); } return (int)(sum/samples); } // Main loop unsigned long lastPublish = 0; void loop() { // Serial commands if (Serial.available()) { String s = Serial.readStringUntil('\n'); processSerialCommands(s); } // Attempt auto-calibration if needed if (!autoCalibrated && (isnan(R0_MQ4) ¦¦ isnan(R0_MQ5) ¦¦ isnan(R0_MQ7) ¦¦ isnan(R0_MQ135))) { tryAutoCalibrate(); } unsigned long now = millis(); if (now - lastPublish < PUBLISH_INTERVAL) { delay(10); return; } lastPublish = now; // read sensors float hum = dht.readHumidity(); float t_dht = dht.readTemperature(); float t_tmp102 = 0.0; // read TMP102 tca_select(0); // Read TMP102 manually (as earlier helper). Minimal checks: Wire.beginTransmission(0x48); Wire.write(0x00); if (Wire.endTransmission(false)==0 && Wire.requestFrom((uint8_t)0x48,(uint8_t)2)) { uint8_t msb = Wire.read(); uint8_t lsb = Wire.read(); int16_t raw = ((msb<<8)¦lsb) >>4; if (raw & 0x800) raw ¦= 0xF000; t_tmp102 = raw * 0.0625; } else t_tmp102 = NAN; int raw_mq5 = readADCaveraged(PIN_MQ5_ADC,6,3); int raw_mq7 = readADCaveraged(PIN_MQ7_ADC,6,3); int raw_mq4 = readADCaveraged(PIN_MQ4_ADC,6,3); int raw_mq135= readADCaveraged(PIN_MQ135_ADC,6,3); int raw_ky_a0= readADCaveraged(PIN_KY_A0,6,3); int raw_pot = readADCaveraged(PIN_SIG_POT,6,3); int ky_d0 = digitalRead(PIN_KY_D0); // ADS1115 optional read (ch0,ch1) tca_select(0); int16_t ads0 = ads.readADC_SingleEnded(0); int16_t ads1 = ads.readADC_SingleEnded(1); // convert -> vModule -> Rs float v_mq4_adc = adcToVolt(raw_mq4); float v_mq5_adc = adcToVolt(raw_mq5); float v_mq7_adc = adcToVolt(raw_mq7); float v_mq135_adc = adcToVolt(raw_mq135); float v_ky_adc = adcToVolt(raw_ky_a0); float v_mq4_mod = vModuleFromAdc(v_mq4_adc); float v_mq5_mod = vModuleFromAdc(v_mq5_adc); float v_mq7_mod = vModuleFromAdc(v_mq7_adc); float v_mq135_mod = vModuleFromAdc(v_mq135_adc); float v_ky_mod = vModuleFromAdc(v_ky_adc); float Rs4 = computeRsFromV(v_mq4_mod, RL_MQ4); float Rs5 = computeRsFromV(v_mq5_mod, RL_MQ5); float Rs7 = computeRsFromV(v_mq7_mod, RL_MQ7); float Rs135 = computeRsFromV(v_mq135_mod, RL_MQ135); // compute ppm if R0 present float ppm4 = isnan(R0_MQ4) ? NAN : computePPM(Rs4, R0_MQ4, CURVE_MQ4); float ppm5 = isnan(R0_MQ5) ? NAN : computePPM(Rs5, R0_MQ5, CURVE_MQ5); float ppm7 = isnan(R0_MQ7) ? NAN : computePPM(Rs7, R0_MQ7, CURVE_MQ7); float ppm135 = isnan(R0_MQ135) ? NAN : computePPM(Rs135, R0_MQ135, CURVE_MQ135); // determine alarm (use ppm if available else raw thresholds) bool alarm = false; const float TH_CH4 = 500.0, TH_LPG = 500.0, TH_CO = 50.0, TH_AIRQ = 300.0; if (!isnan(ppm4) && ppm4 > TH_CH4) alarm = true; if (!isnan(ppm5) && ppm5 > TH_LPG) alarm = true; if (!isnan(ppm7) && ppm7 > TH_CO) alarm = true; if (!isnan(ppm135) && ppm135 > TH_AIRQ) alarm = true; // fallback raw if (raw_ky_a0 < 1200) alarm = true; if (ky_d0 == LOW) alarm = true; if (alarm) { digitalWrite(PIN_LED, HIGH); digitalWrite(PIN_BUZZER, HIGH); } else { digitalWrite(PIN_LED, LOW); digitalWrite(PIN_BUZZER, LOW); } // Build JSON payload String ts = isoTimestamp(); // Build payload manually (avoid String concat explosion) - but okay for moderate size char buf[1400]; snprintf(buf, sizeof(buf), "{" "\"ts\":\"%s\",\"device\":\"%s\"," "\"dht_temp\":%.2f,\"dht_hum\":%.2f,\"tmp102\":%.2f," "\"mq4_raw\":%d,\"mq5_raw\":%d,\"mq7_raw\":%d,\"mq135_raw\":%d," "\"mq4_rs\":%.2f,\"mq5_rs\":%.2f,\"mq7_rs\":%.2f,\"mq135_rs\":%.2f," "\"mq4_ppm\":%.2f,\"mq5_ppm\":%.2f,\"mq7_ppm\":%.2f,\"mq135_ppm\":%.2f," "\"ky_a0\":%d,\"ky_d0\":%d,\"ads0\":%d,\"ads1\":%d,\"pot\":%d,\"alarm\":%d" "}", ts.c_str(), DEVICE_ID.c_str(), isnan(t_dht)?NAN:t_dht, isnan(hum)?NAN:hum, isnan(t_tmp102)?NAN:t_tmp102, raw_mq4, raw_mq5, raw_mq7, raw_mq135, Rs4, Rs5, Rs7, Rs135, isnan(ppm4)?-1.0:ppm4, isnan(ppm5)?-1.0:ppm5, isnan(ppm7)?-1.0:ppm7, isnan(ppm135)?-1.0:ppm135, raw_ky_a0, ky_d0, ads0, ads1, raw_pot, alarm?1:0 ); String payload = String(buf); // 1) Append to /devices/<DEVICE_ID>/history (POST to create unique entry) String histPath = "/devices/" + DEVICE_ID + "/history"; String resp = firebasePost(histPath, payload); // optionally parse response to get key (not required) // 2) Update latest snapshot via PUT String latestPath = "/devices/" + DEVICE_ID + "/latest"; bool okLatest = firebasePut(latestPath, payload); // Serial debug Serial.println("Published:"); Serial.println(payload); delay(10); }

### Important Notes

- Ensure to replace placeholders such as YOUR_SSID, YOUR_PASS, YOUR_PROJECT_ID, and YOUR_DB_SECRET_OR_TOKEN with actual values for WiFi and Firebase configuration.

- The code requires specific libraries, including Adafruit_ADS1015, DHT, and others, which must be installed in the Arduino IDE.

- The ESP32 code includes functions for reading and processing sensor data, managing network connections, and handling serial commands.